- State Faraday’s law of Electromagnetic induction.

- State Lenz’s law of electromagnetic induction.			(2mks)

- Define the volt.								(1mk)

- State three factors affecting the size of the induced voltage.	 (3 mks)

- State one source of mains electricity					(1mk)

COILS

- An armature composed  of turns of insulated copper wire wound on a laminated soft- iron core is rotated in a magnetic field to generate an e.m.f. Use this information to answer the following questions.

  - State two factors other than the speed of rotation that affect the

magnitude of the e.m.f generated.

  - State the reason why soft- iron is laminated.

N

S

The free ends of a coil are connected to a galvanometer. When the north pole of a magnet is moved towards the coil, the pointer deflects towards the right as shown. State with reason the behaviour of the pointer in the following cases.

- The north pole of the magnet is held stationary near p.

- The south pole of the magnet is made to approach the coil from Q.

- In the figure below, the bar magnet is moved out of the coil

X

I

(i) If the current, I is induced in the coil in the direction shown, what is the polarity of x of the magnet?						(1mk)

(ii) Explain briefly the source of electrical energy in the circuit	(1mk)

- Figure below shows a rigid circuit ABCD suspended by a silk thread from a support. The coil AB is made of copper.

Silk thread

Motion

N

S

B

A

The magnet near A is suddenly pulled to the left. State and explain the  observation on the circuit. 							(2mks)

- The figure below shows ends of a solenoid connected to a galvanometer as shown below and a magnet plunged into the coil.

G

N

S

Show the direction of induced current                               (2mk)

- A coil  x is moved quickly away from the end of  a stationery Magnet Y and

current flows as show n in

Y

Motion

State the polarity of Y						(1mk

- Figure shows two coils P and Q placed close to each other.

- When the switch k is closed, an e.m.f is induced in coil Q. Similarly, an e.m.f is induced in coil Q when the switch K is opened.

- Explain why the induced current in coil Q is higher when the switch K in coil P is opened than when it is closed. 					(2 mk)

- With the help of diagrams, show the direction of the induced current in coil Q  when the switch k is closed and when it is opened.  (2 mk)

- Which phenomenon is being demonstrated in this set up? 	(1mk)

- Suggest a way in which the induced e.m.f in the secondary coil Q can be increased. 										(1mk)

- Figure  shows two coils connected using a soft iron core.

A

B

X

Y

When the switch is closed, the galvanometer deflected in the direction shown. Determine:

    - The magnetic poles X and Y					(2 mk)

    - The electric polarity A and B of the cell.				(2 mk)

    - Compare the deflection of the galvanometer when the switch is closed and when its opened. 								(2 mk)

- The figure below shows mutual induction

C1

C2

A

(a) Show the deflection in C2 when the switch for C1 is closed	 (1mk)

(b) Using a diagram illustrate one way of increasing the e.m.f induced with the 	      same number of cells and coils.				(2mks)

- The diagram below shows an induction coil used to produce sparks.

B

A

C

(i) Name parts labeled A, B and C 				(3mks)

A…………………………………………………………………

B…………………………………………………………………

C…………………………………………………………………

(ii) Briefly explain how the induction coil works. 	(4mks)

- Fig shows an induction coil.

Gap

D

A

C

B

S

Contacts

Soft iron armature

i) Name the parts labelled A, B, C and D.					(2mk)

ii) Briefly explain what happens from the time the switch‘s’ is closed onwards. (4mk)

iii) State the purpose of the part labelled D in the circuit.		(1mk)

GENERATOR

- If a “boda boda” cyclist wants his head lamp to light more brightly what must he do?

- The figure below shows certain machine

N

S

Output

B

A

Axis of rotation

(i)	State the name of the machine shown in the diagram above. (1mk)

(ii)	What are the names of the parts labeled A and B	(2mk)

(iii)	What would be the effect of doubling the number of turns on the coil if the speed of rotation remained uncharged?		(1mk)

(iv)	Which of the output terminals is positive at this instant if the coil is rotating in the direction shown in the diagram (anticlockwise)?(1 mk)

(v)	On the axes below sketch a graph to show how potential difference across R varies with the inclination angle. The coil is initially horizontal. 	  (1mk)

E.M.F  (v)

θ degrees

(vi)	Give reasons for the changes in the emf as the coil rotates from 00 to 1800 3mk

Coil

N

S

Slip rings

Brushes

Load

Figures shows a generator of electricity

The coil rotates at a steady speed of 60 turns rotations per minute. An oscilloscope is connected a cross the load and adjusted to get on the screen a trace for one rotation of the coil. Sketch the trace for one rotation starting and finishing in the same position of the coil as shown in the figure.      (3mk)

- (i)	Figure below shows an electric generator. The points P and Q are connected to a cathode Ray oscilloscope (C.R.O)

Axis of rotation

Q

P

N

S

Commutators

Brush

(ii)	Sketch on the Axis provided the graph of the voltage output  as seen on the C.R.O, given that  when t= o, the coil is at the position shown in the figure 2mk

Voltage (V)

Time (s)

A

C

D

B

Y

N

S

M

Y

Axis of rotation

K

The diagram below shows a simple d.c generator.

i) 	Name the part labeled X and Y    						(2mk)

ii) 	The coil is rotated  in Anticlockwise direction  indicated  using an arrow on 		the figure the direction of the induced current as the arm CD passes the position

MK as shown.  						(1mk)

iii)	 Sketch on the axis below a graph of the e.m.f generated against the angle as

arm  CD rotates from 90o to 180o

E.M.F

Angleϴ

900

1800

N

S

A single coil of wire is held between the poles of a magnet as shown in figure  below. The ends of the coil are connected to a sensitive centre-zero galvanometer. The coil is held out of the field of the magnet, the galvanometer deflects momentarily to the left.

(a)	Explain why the galvanometer deflects 				 (1mk)

(b)	Explain how the deflection would be made larger with the same apparatus(2mk)

(c)	For each of the following cases, describe what is observed on the galvanometer giving reasons

(i)	Coil moved towards south pole without rotating			(2mks)

(ii)	Coil rotated through 900 in clockwise direction			(2mks)

- Figure shows a cross –section of a bicycle dynamo. The wheel is connected by an axle to apermanent cylindrical magnet and is rotated by the bicycle tyre.

Wheel

N

S

Coil

Axle

Soft iron

(i) Explain why the bulb lights			(1mk)

(ii) How can the bulb be made brighter?(1mk)

- Graph A below shows how the e.m.f produced by a simple dynamo varies with time. Graphs Band C show how the e.m.f produced by the same dynamo varies with time after certain alterations and modifications have been made

+

Time (s)

P

S

0.1

0.2

W

E.m.f

Q

O

R

T

U

V

-

A

Time (s)

E.m.f

O

-

B

C

+

-

E.m.f

0.1

0.2

0.1

0.2

O

Time (s)

(i)	What is the frequency of the alternating e.m.f as shown by graph A? (2 marks)

(ii)	Which letters on graph A correspond to the plane of the coil of the dynamo being parallel to the magnetic field?						(1 mark)

(iii)	Explain why the e.m.f at Q is zero					(3 marks)

(iv)	What alterations has been made for the dynamo to produce the e.m.f represented by graph B?								(1 mark)

(v)	What modification has been made to the dynamo for it to produce the e.m.f represented by graph C?						(1 mark)

(vi)	A dynamo is driven by a 5kg mass which falls at a steady speed of 0.8m/s. the current produced is supplied to a 12W lamp which glows with normal brightness. Calculate the efficiency of this arrangement				(2 marks)

TRANSFORMERS

- State one difference between a transformer and an induction coil.  (1 mk)

- Describe briefly the energy changes involved in the generation of electrical energy at a hydropower station.                                           (2mk)

- Explain why the core of a transformer tends to get hot when in use.(2mk)

- In what form is energy lost in a cable during transmission.	(1 mk)

- Give any two (2) structural features in a transformer design which help in achieving high efficiency. 						 (2mk)

- What are eddy currents						(1mk)

- State the reason why soft iron it laminated in transformers.	 (1mk)

- Explain why in a transformer, it is alternating current which is fed to the primary coil and not the direct current. 					(1 Mark)

- State the advantage of generating a.c rather than d.c voltage in a
power station.			(1mk)

- Give the structural features in a transformer design which helps in achieving high efficiency.     							(3mk)

- State one cause of energy losses in a transformer and explain how it can be minimized.	      (2mk)

- State one way in which power is lost in a transformer. 		(1mk)

- The core of a transformer is made of soft iron which is a ferromagnetic material.

(i)     Name one other ferromagnetic material. 			(1mk)

(ii)    State two features of the soft iron core that helps to improve the efficiency of the transformer. 								(2mk)

- One of the factors that affect efficiency of a transformer is hysteresis losses. What is Hysteresis losses							(1mk)

- State two energy losses in a transformer 				(2mk)

- What are the advantages of transmitting power at

- Very high voltages

- Alternating  voltage

- How are the high voltages acquired during power transmission achieved?												(1mk)

- Explain why the voltage of mains electricity has to be stepped up immediately after power generation

- In the design a step up transformer;

(i) Which coil would be made with thick conductors 			1mk

(ii) Explain your answer in c (i)						1mk

- A large substation transformer is used to step down voltage from 11,000V to 450V.

i)	Determine the ratio of the turns in the primary to secondary coils.

ii)	How is the efficiency of this transformer ensured?

iii)	State one function of the core in a transformer.

- The diagram below shows two coils A and B wound on a soft iron core.

240 V

B

500

turns

Soft iron core

A

1000 turns

A

A has 1000 turns and B has 500 turns and a resistance of 100Ω. Determine the maximum current measured by the ammeter. 	    (4mks)

- Two coils T and S are wound on a soft iron core as shown. T has 1000 turns while S has 600 turns and resistance of 100

250 V

a.c

S

T

A

Calculate the maximum current measured by the ammeter.

- A 6v, 24w lamp shines at full brightness when it is connected to the output of this main transformer as shown in the figure.

240 V

800 Turns

6 V

24 W

Assuming the transformer is 100% efficient, calculate

- The number of turns in the secondary coil if the lamp is to work at its normal brightness                                                   (2mk)

- The current which flows in the main cables.           (2mks)

- Figure below represents transformer connected to an a.c. source and a resistor R.

VP

VS

IS

IP

Compare the magnitude of:

(i) Voltage Vp and Vs									(1mk)

(ii) Current Ip and Is									(1mk)

- Figure below shows a transformer which is 90% efficient.

240V

12V, 36W

(i)    Determine the number of turns in secondary coil if the number of turns in the primary coil is 4000.							(2mks)

(ii)     Determine the current in the primary coil if the bulb is operating normally										(3mks)

(iii)    Explain why long distance power transmission is done at a very high voltage     									(2mks)

- Figure below shows a single phase demonstration transformer intended to converted 24V, 50Hz as AC supply to 240V, 50Hz

240 V

24 V

50Hz

Soft iron core

50Hz

Primary coil

Secondary coil

(i)	What is the purpose of the soft iron core?				(1mk)

(ii)	If the primary core has 50 turns of coil, how many turns of coils should the

secondary have?								(2mks)

RL=4Ω

A

240 V

a.c

S

NP

v

NS

NP=800

NS=40

The circuit below shows a step-down transformer used to light a lamp of resistance 4Ω.  Calculate.

i)	The reading of voltmeter, V, with ,S, open.		(2mks)

ii)	The reading of the ammeter A with S closed (neglect the effective resistance of the secondary winding)						(2mks)

iii)	The power dissipated in the lamp.					(2mks)

iv)	The primary current if the transformer is 90% efficient.	 (3mks)

- A transformer of 480 turns in the primary coil used to connect a 9-volt a.c. electric device to a 240V a.c. mains power supply.  Calculate the number of turns in the secondary coil

- A transformer in a welding machine supplies 6 volts from a 240V mains  supply. If the current used in the welding is 30A. Determine the current in the mains.

- A generator produces 150kW at a voltage of 5 kV. The voltage is stepped up to 60kV and transmitted through cables  of resistance 150 to a step – down transformer in a substation. If both transformers are 80% efficient,

Calculate the:-

(i)	Current through the transmission cables.				(3mks)

(ii)	Power lost during transmission.						(3mk)

- A transformer with 1200 turns in the primary circuit and 120 turns in the secondary circuit; it produces heat at the rate of 600w. Assuming 100% efficiency, determine the:

i)	Voltage in the secondary circuit.			(2mk)

ii)	Current in the primary circuit.			(2mk)

- A student designed a transformer to supply a current of 10A at a potential difference of 60V to a motor from an A.C mains supply of 240V.  If the efficiency of the transformer is 80%, determine the;

a)	Power supplied to the transformer

b)	Current in the primary coil.

- An ideal transformer has 500 turns in the primary circuit and 2000 turns in the secondary circuit. When the primary circuit is connected to a 200V a.c. source, the power delivered 	to a resistor in the secondary circuit is formed to be 1000W.

(i)  Giving a reason compare the thickness of the wires used in the primary and secondary circuit.					(2mk)

(ii) Determine the current in the secondary circuit			(3mk)

(iii) Determine the current in the primary circuit.			(3mk)

- A Transformer has 800 turns in the primary winding and 40 turns in the secondary winding.  The alternating e.m.f connected to the primary is 240V and the current is 0.2 A.Find;

i)  Secondary e.m.f

ii)  The power in the secondary if the transformer is 90% efficient.(3mk)

- A hydro-electric power station produces 500KW at a voltage of 10KV. The voltage is then stepped up to 150KV and the power is transmitted through cables of resistance 200Ω to a step down transformer in a sub-station. Assuming that both transformers are 100% efficient. Calculate;

(i) The current produced by the generator				(2mks)

(ii) The current that flows through the transmission cables	(2mks)

(iii) The voltage drop across the transmission cables			(2mks)

(iv) The power loss during transmission 				(2mks)

(v) The power that reaches the sub-station				(2mk)

- A transformer is designed to supply a current of 12A at a p.d. of 80V. The inlet cable is to be connected to an a.c. mains of 240V. The efficiency of this transformer is 80%. Calculate:

(i)	Current in the primary coil of the transformer		(2mks)

(ii)	The power supplied to the transformer 			(3mks)

(iii)	Explain how energy losses in a transformer are reduced by having a soft iron core.							(2mk

- A transformer is designed to supply a current of 5A at a potential difference of 50V to a motor from an a.c supply of 240V. If the efficiency of the transformer is 80%. Calculate

(i)	The power supplied to the transformer			(3mk)

(ii)	The current in the primary coil				(2mk)

- A transformer uses 240 V a.c supply to deliver 9.0A at 80.0V to a heating coil. If 10% of the energy taken from the supply is lost in the transformer itself, what is the current in the primary winding? 			 (3mks)

- A transformer of 600 turns in the primary circuit and 9000 in the secondary circuit is connected to a240V mains supply. The current in the secondary circuit is 0.15A. What is the current in the primary circuit? (Assume 100% efficiency)												(3mks)

- A transformer has 960 turns in its primary coil and n turns in its secondary coil and is connected to a 240V supply. Given that the transformer is 80% efficient and it is used to operate a 6V, 24W bulb. Find

(i)       The number of turns in its secondary coil.                    (2mk)

(ii)       The current flowing in the primary coil.      (2mk)

- A transformer was assigned to operate a 60W, 12V lamp when it is connected to a 240vmains supply. Calculate the current that would be drawn from the mains if it is 80% efficient.  						 (4mk)

- A 240v mains transformer has 800 turns in the primary and N turns in the secondary.  It is used to supply energy to a 12V, 21W lamp.

i)	Calculate the number of turns in the secondary.		(2mks)

ii)	What is the efficiency of the transformer if the current drawn from the 240V mains 	is 100 mA?					(2mks)

- A transformer has 10,000 turns on its secondary coil and 100 turns on its primary coil. An alternating current 5.0A flows in the primary circuit when it is connected to a 12v a.c supply.

i) 	State the type of transformer 						(1mk)

ii) 	Calculate the power input to the transformer				(3mk

iii) Calculate the E.M.F across the secondary coil. 			(3mk)

iv) Determine the maximum current that could flow in  a circuit connected to the secondary 	coil if the

v)	In transmitting power why is it necessary to step it up before transmission. 												(1mk)

- A transformer with primary coil of 400 turns and secondary coil 200 turns is connected to 240ac mains.

(i) Calculate the secondary voltage.						(2mk)

(ii) If the primary current is 3.0A and secondary is 5.0A. What is its efficiency?									(2mk)

(iii) How is energy loss in transformer due to hysteresis minimized.(1 mk)

(iv) Explain why electrical power is transmitted at very high voltages.(1mk)