- Explain why mains electricity is transmitted through alternating current and not direct Current							(2mk)

- State the energy transformation that takes place in a hydroelectricpower station.

- Electrical energy is distributed as alternating current (a.c) at high voltage.

Explain why:

(i)	A very high voltage is necessary.					(1mk)

- Thick aluminium transmission lines are recommended to carry the current.

(iii)	The power is transmitted as Alternating voltage (a.c)(1mk)

- State two dangers of high voltage transmission.			(2mk)

- What is the purpose of having a national grid in power transmission.	(1mk)

- State two factors which affect heating in electric circuits.

- A man driving a car in rain discovers that the moment he alights from it, while touching its body he gets an electrical shock. Why was he not getting the shock while inside even if he touches metallic parts?

- Name a device used to change light energy directly into electrical energy.

THREE PIN PLUGS

- Explain why three pin plugs are preferred to two pin plugs in the use with iron box							(3mks)

- Explain the letters L, N and E usually marked on the three pin plug	(3mk)

- State two reasons why the earth pin is normally longer than the other two pins.

- Give two advantages of a circuit breaker instead of a fuse in domestic wiring

- Distinguish between a fuse and a circuit breaker 			1 mk

- State the importance of a fuse in a circuit and explain how it works.(2mk)

- What property does a fuse wire have that makes it suitable for controlling excessive current in circuit?

- Figure below shows a typical example of a fuse.

Fuse wire

What modification can be done to the fuse wire to give it a higher rating?(1mk)

A

Heater

B

C

Drier

Figure below shows part of a ring main circuit connected to the hair drier in a salon

Identify the parts A,B and C

What modification can be done to a fuse wire to give it a higher rating?

- Fig shows part of a domestic wiring system with a fault. Identify the fault and draw a diagram with the correction.

Appliance

L

N

- The figure shows the electric wiring of an electric cooker X, Y and Z are main wires.

Cooker casing

X

Y

Z

Identify Y and X giving reasons.

- Study the figure below:-

B

D

A

C

Cable

i)	What name is given to the fitting in the diagram.		(1mk)

ii)	Identify the parts labelled A, B, C and D				(2mk)

iii)	State the colours A, B and C.						(3mk)

- Figure below shows a flex to the 13A – 3pin.

Fuse

Blue insulation

Brown insulation

Yellow /green insulation

(i)	List two mistakes and suggest corresponding remedies.         (4mks)

(ii) 	Why would it be wrong to fit an electric fire in a bathroom on the wall directly

above the bath?			         (1mk)

(iii) What would happen if this plug was connected to the mains of the socket.(1mk)

- The fig 4 below shows a section of a circuit in domestic wiring.

N

L

5A

Main switch

Heating element

30A

Identify one defect in the wiring.				     (1mk)

- Figure shows a modern mains appliance

Appliance

L

N

E

(i)	State the purpose of lead E						(1mk)

(ii)	Explain why the fuse is connected to the live wire		(2mks)

DOMESTIC WIRING

- In domestic ring there are various circuits which may be used. Name three											(3mk)

- What is normally contained in a consumers control unit in a domestic wiring system	(1mk)

- Explain why bulbs in a long corridor are not wired in series.	(2mk)

- An electrician installed electric wiring in a house and connected the bulbs and the switches as shown in figure below.

To mains

S1

S2

S3

(a) 	State what happens when switch:				(1mk)

I	S1 only is closed

II	S2 only is closed							(1mk)

III	S3 only is closed							(1mk)

IV	All the switches are closed.					(1mk)

(b) 	Redraw a diagram showing the best position the bulbs should be installed.

(c)	Explain why you consider this your arrangement to be the best. 	(2mks)

CALCULATIONS

- An electric iron rated 240V, 750w is to be connected to a 240v mains supply through a 3A fuse.  Determine whether the fuse is suitable or not. 	(3mk)

- An 8000w heater is used for a total of 1.5 hours a day. Calculate the cost of using this shower for a month of 30days if the cost of an electrical unit is ksh. 5.40.		2mk

- A heater is marked 3kW, 240V. The fuses available are marked 10A, 13A and 20A. Which fuse is most suitable?				(3mks)

- An electric heater rated 240V, 3000V is to be connected to a 240V mains supply, through a 10A fuse.  Determine whether the fuse is suitable or not.

- What current will a 500 resistor connected to a source of 240V draw?

- A house has a lighting circuit operated from a 240 volts mains supply. Three bulbs rated 100w 240v, ten bulbs rated 40w 240v and eight bulbs rated 60w 240v are switched on at the same time. What is the most suitable fuse for this circuit?	                 (3mks)

- When a current of 2.0A flows in a resistor for 10 minutes, 15000 Joules of electrical energy is dissipated.  Determine the voltage across the resistor.

- An electric bulb rated 40W is operating on 240v mains. Determine the resistance of its filament.

- A house has five rooms with 240V, 60W bulbs. If the bulbs are switched on from 7.00 p.m to 10.30 p.m.

(i) Calculate the power consumed per day in kilowatt-hours. 	(3mks)

(ii) Find the cost per week for lighting these rooms at Ksh. 6.70 per unit.(3mk)

- How many 100W electric irons could be safely be connected to a 240V mains circuit fitted with a 13A fuse?

- Determine the largest number of 75W bulbs which can be safely used to run on a 240V source with a 5A fuse.				(3mk)

- Find the maximum number of 75W bulbs that can be connected to a 13A fuse on a mains supply of 240V.

- Calculate the power of a devise which has a p.d of 250V applied across it when a current of 0.5A passes through it.

- An electric heater takes a current of 12.5A from 240V power supply.Calculate

a) Its power rating.

b) Its resistance

- A 75W lamp and a 1.5kW water heater are connected in turn to a 240V supply.

(i) 	Calculate the current in each appliance. 			(3mks)

(ii) 	Explain the difference you would expect to find between the wiring used to connect the bulb and the water heater to the mains supply. 										(2mks

- A power line from a power substation to a town some distance away, has a resistance of 0.10 Ω per kilometer. Determine the rate of energy loss in the transmission of power over 50km at a current of 60 Amperes	 (3mks)

POWER COST

- An electric bulb rated 12V, 36W allows current to pass through for 1hour. How much electrical energy is dissipated by the bulb?		(2mks)

- An immersion heater rated 1.5KW is used continuously for 30min per hour per day. Calculate the cost per week per ksh 6.70 per unit.	(3mk)

- An immersion heater rated 300W is used continuously for 45 minutes per day.  Calculate the cost per week at 60cts per unit.

- A 50w bulb is used continuously for 36 hours. Determine the cost of energy consumed at a cost of Kshs. 2 per unit.

- A 60W bulb is used continuously for 36 hours. Determine the energy consumed, giving your answer in kilowatt hour (kWh)

- Determine the cost of using an electrical iron box rated 1500W, for a total of 30 hours given that the cost of electricity per kWh is Kshs. 8.

- Electrical energy costs Kshs. 1 per kWh unit.  Find the cost of using an electric heater of power 7.5kW for a day.

- An electric iron box is rated 2500W and uses a voltage of 240V.  Given that electricity costs Kshs. 1.10 per Kwh, what is the cost of using it for 6 hours? What is the cost of operating all the parts at once for 30 minutes if electricity costs Sh.10.00 per unit?

- An electric cooker has an oven rated 2500w, a grill rated 1500w and two plates each of 1000w.  This cooker operates directly from 240v supply. What is the cost of operating all the parts at once for 30 minutes if electricity costs Sh.10.00 per unit?	(3mks)

- Four 40W bulbs and six 100W bulbs were switched on for 5 hours a day for domestic use in a certain institution. Find the monthly bill for the consumer given that the cost of electricity in the country is at Sh. 5.50 per unit (Take 1 month = 30 days and standing charge of Sh. 150)			(4mks)

- A plug is connected to a hair dryer rated 100W, 240V A.C If the hair dryer runs for 3 hours per day for 7 days, calculate the.

(i) 	The no. of kWh used 						(2mk)

ii) 	The cost of electricity paid at a rate of ksh. 5.50 per unit	(3mk)

- Calculate the cost of using the following appliances in one month (30 days) of the company rate is Ksh.9.50 per unit.

- A 2000W water heater for 2 hours per day.

- A 75W bulb for 10 hours per day.

- An 1500W electric iron for 1 hour per day.

- A standing charge of Ksh.200.

- Fuel cost levy at 70 cents per unit.					(4 mk)

- A 2kW electric heater is used for 10hrs. Calculate the cost of electricity if it costs 30ksh/ unit							(3mks)

- An immersion heater is rated 5 KW 250V. It is used for 2 hours daily. If electricity cost sh12.00 per unit, calculate the weekly cost of running the heater.                      (2mk)

- An electrical heater of power 2.5kW is used everyday for 45min. Calculate the cost electrical energy consumed in Kilowatt hours in a month of 30 days if the cost per unit issh16.50 	(2 mk)

- A student has a large number of 240V, 60W coloured bulbs he wishes to use for decorations so that the bulbs operate normally.

(i)	How many bulbs can be connected to a 240V supply through a 5A fuse?							(2mk)

(ii)	If electric energy cost Kshs. 3.00 per unit, what will be the cost of running the above circuit for 5 hours a night for 20 nights?(2mk)

- Suppose you have the following appliances for use in your house.

| Appliances | Power rating (W) | Time used hours/day |
| --- | --- | --- |
| Cooker TV set Electric kettle Radio. | 5000 100 1500 50 | 2½ 3 ½ 8 |

Calculate the cost of electrical energy consumed in Kilowatt hours in a month of 30

daysif the cost per unit issh12.60

SCHEEM

- What is the purpose of having a national grid in power transmission.(1mk)

ANS	The national grid helps to ensure if one power station breaks down there would still be power everywhere (no black out)