.

- Define the term moments.

- State the principle of moments.

- Define the term moment of a force.

- Name four activities which produce a turning effect

- Why is it very difficult to open a door from a point too close to hinges?

- Explain why it is difficult to steer a bicycle by gripping the centre of the handlebars.    (2mks)

- A load of 900N is placed 3m from a pivot. Calculate the moment due to the load.

- A girl of mass 60kg sits 4m from a pivot. Calculate the moment due to the girl.

- Calculate the moment due to the force F below.

Pivot

F = 45N

80 cm

- A uniform meter rule pivoted at its centre is balanced by a force of 100N at 20cm and another force of F at the 75cm mark.

50cm

100N

F

75cm

20cm

0

100cm

- Calculate the force F.

- What is the reaction at the pivot?

- Three forces are applied on a meter-ruler as shown.

50cm

72N

F

90cm

70cm

0

100cm

30N

- Calculate the force F.

- What is the reaction at the pivot?

- The figure below shows three forces applied on a uniform metal rod of length 20m.

0

10m

90N

240N

16m

X

2m

20m

300N

Calculate the position X where the force of 240N is placed.

- The figure below shows a meter rule balanced by four forces at its centre.

5cm

30cm

80cm

90cm

100cm

175 N

200N

50 N

W

50

- Determine the weight W.

- What is the reaction at the pivot?

- The figure below shows a uniform  balanced by four forces at its centre. Determine the value of force F.

0cm

20cm

50cm

75cm

90cm

100 N

200 N

150 N

F

100cm

- A uniform wooden plank of length 50m is pivoted at its centre and balanced by a force of 150N at 15m mark and another force of 60N on the other side at a distance D from the pivot.

150N

60N

D

15m

0

50m

Determine the distance D

- The diagram in figure represents a system in equilibrium.

65cm

Centre of gravity

50cm

3kg

20cm

38N

F

Determine the force, F needed to keep the system in equilibrium.

40cm

T

35cm

8N

4N

10cm

The figure below shows a uniform metal rod balanced at the centre by different forces.

Determine the value of T.									(3mk)

Pivot

40N

10 cm

100N

60 cm

F

0 cm

80 cm

A uniform meter ruler is suspended vertically form a pivot at the 0cm mark and maintained vertically by three horizontal forces acting at the 10cm 60cm and 80cm as shown below. Calculate the force F acting at the 80cm mark.

- Figure below shows a uniform rod AB of negligible weight pivoted at A.

20cm

A

W

30cm

35cm

2 kg

3 kg

B

If the system is in equilibrium, determine the weight W shown in the diagram.

- The figure below shows a uniform ruler balanced at the centre due to action of some forces as shown.  A is a magnet of weight 30N and B is a permanent magnet fixed on to the bench.

B

50cm

75cm

0

100cm

10 cm

30N

80N

A

- Determine the force between A and B.

- State the nature of the magnetic force between A and B.

- The figure below shows a uniform light rod balanced due to action of two forces shows.  G is a magnet of weight 3N and H is a permanent magnet fixed on to the bench.

H

50cm

80 cm

0

100cm

35 cm

3N

5N

G

- Determine the force between G and H.

- State the nature of the magnetic force between G and H

- Figure shows a hydraulic press system using a lever of negligible mass on the side of a small piston pivoted at point P. A force of 120N is applied at R.

P

100 cm

50 cm

Liquid

Area= 240cm2

A Bale

120 N

A =60 cm2

R

(i)    Calculate the force F exerted by small piston on the liquid.

(ii)   Find the weight of the Bale supported by the large piston.

- The figure below shows two equal and opposite forces acting on a meter at the 15cm mark and 75cm marks respectively. If each of the forces has a magnitude of 80N, calculate the moment on the meter rule about 35cm mark.

P

15 cm

80 N

80 N

75 cm

0 cm

100 cm

35 cm

P

22 cm

60 N

60 N

67 cm

0 cm

100 cm

42 cm

The figure below shows two equal and opposite forces acting on a meter at the 22cm mark and 67cm marks respectively. If each of the forces has a magnitude of 60N, calculate the moment on the meter rule about 35cm mark.

- A uniform meter rule is pivoted at the 30cm mark by a force of 16N placed at the 0cm mark.

30cm

16N

0

100cm

Calculate the weight of the meter rule.

- The figure below shows a meter rule in equilibrium.

20cm

120N

80cm

0

100cm

25N

- Calculate the weight of the meter rule.

- What is the reaction at the pivot?

- The figure below shows a uniform meter rule of weight 1N with two weights of weight 0.18N and 0.12N suspend from its ends.

W = 1N

0.18 N

0.12 N

50

0

100 cm

Determine how far from the 0.18 N weight a pivot should be placed in order to balance meter rule.							(3mk)

100cm

500g

0 cm

26cm

35cm

Metre rule

A metre rule whose centre of gravity is at the 50cm mark balances at the 35cm mark when a mass of 500g is placed at the 26cm mark as shown the below

Determine the mass of the metre rule.

0cm

160N

60N

10cm

100cm

30cm

X

The figure below shows a uniform wooden plank of length 80m and weight 40N balanced at the 30m mark as shown. Determine the reading at position X where the 60N weight is placed.

- The figure below shows a uniform metal rod of length 20m and weight 25N in equilibrium.

0

10m

F

240N

6m

2m

20m

100N

Calculate the force F.

- The figure below shows a meter rule balanced by four forces at its centre.

10 cm

60cm

80cm

W

100cm

80N

50 N

120N

Determine the weight W of the ruler

- The figure below shows a meter rule balanced by four forces of 80N, 150N, 120N, and 50N. Determine the weight of the ruler.

50cm

10 cm

60cm

80cm

150N

100cm

80N

100N

120N

- The figure below shows a uniform metal bar of length 50m in a balanced condition. Determine the weight of the bar.

0m

35m

40m

50m

200 N

100 N

- Figure below shows a uniform rod AB of weight 20N pivoted at A.

4m

A

W

3.5m

4.5m

40N

80N

B

If the system is in equilibrium, determine the weight W shown.

- The figure below shows a uniform half meter rule of weight 24N balanced at the 15cm mark.  K is a magnet of weight 60N and M is a permanent magnet fixed on to the bench.

M

15cm

0

50cm

9 cm

60N

K

- Determine the force between magnets K and M.

- State the nature of the magnetic force between K and M.

- Fig shows a system in equilibrium with the horizontal rate.

40cm

P

Q

X

A

B

80N

20 cm

String

AB is a uniform rule of length 1.0m and weight 20N.  Calculate the

- Weight of block X

- Tension in the string PQ.

- A uniform metallic bar of length 100cm and mass 40kg is supported horizontally by two vertical spring balances A and B as shown below.

Balance A is 20cm from one end while balance B is 30cm from the other end. Find the reading of each individual balance.

- The figure below shows a uniform plank of weight 20N and length 6m balanced by a 0.5kg mass at a distance X from the pivot point O.

O

X

0.5 kg

Determine the value of X

- A uniform plunk of wood weighing 50N and of length 5m is suspended by two ropes, A and B. A is at 2m from one end while B is at 1.5m from the other end as shown below. A concrete block of weight 100N is suspended from the centre of the plunk

B

100N

2 m

1.5 m

A

1.5 m

Calculate the tension on the string A

- The figure shows a uniform half metre rod that is balanced over a pivot using a block of weight 2N and a spring

5cm

Spring Tension 9N

10cm

5cm

30cm

Block

Given that the tension in the spring is 9N, determine the weight of the rod.

- Figure shows a uniform bar of length 1.0 m pivoted near one end.  The bar is kept in equilibrium by a spring balance as shown:

Given that the reading of the spring balance is 0.6 N, determine the reaction force at the pivot.

- The figure below shows a uniform meter rule of weight 2N kept at equilibrium by a vertical spring balance hung at 20cm mark. A weight of 3N is hung at 100cm mark and the other end pivoted by a hinge at 0 cm as shown.

2N

3N

0cm

2O cm

Spring balance

100cm

Calculate the reading of the spring balance.

- The figure below shows a uniform metre rule which is pivoted at 30.0cm mark. The spring balance is fastened at the 100cm mark and it is at equilibrium when the spring balance records 1.2N. Determine the weight of the metre rule.

0cm

3O cm

Spring balance

100cm

- The figure below shows a uniform meter rule balanced at the 30cm mark by the forces shown. Determine the weight of the meter rule.		(4mk)

120N

30N

10cm

80cm

100cm

30cm

0cm

- The figure below shows a meter rule balanced by four forces at its centre. Determine the value of force F.					(4mk)

0cm

20cm

50cm

75cm

90cm

100 N

200 N

80 N

F

100cm

- The figure shows a regular solid cone in equilibrium

50N

3m

Pivot

12m

Determine the weight of the cone.

- Figure 2 show a solid cone which has a uniform density in equilibrium

5N

0.2m

Pivot

0.9m

Determine the weight of the cone						(3mk)

- The figure below shows force F1 and F2 acting on a metre rule such that it is in equilibrium.

F1

F2

Mark on the figure a third force F3 acting on the rule such that the equilibrium is maintained.

- The figure below shows a uniform metre rule of weight 1.0N suspended from spring balances. A load is attached to the extreme right hand end C. The spring balance attached to the extreme left hand end of the rule (A) reads 0.25N. The spring balance attached at B a distance X from the right hand end reads 1.25N.

0.25N

1.25N

A

B

L

C

X

(i)	Calculate the weight of load L				(2mks)

(ii)	Determine the value of distance X by taking moments about A    (3Mks)

A

Plate

2m

1m

F = 50N

The figure below shows a metal plate 2 m long, 1m wide and negligible thickness. A horizontal force of 50 N applied at point ‘A’ Just makes the plate tilt.

Calculate the weight of the plate.					 (3mk)

- The figure below shows a wheel of mass 10kg and radius 1 m being pulled by a boy against a step 0.4 m high. What force is just sufficient to turn the wheel so that it will rise over the step

0.4 m

Boy

- Figure shows a hydraulic press system using a lever of negligible mass on the side of a small piston pivoted at point P. A force of 400N is applied at R.

P

80 cm

20 cm

Liquid

Large piston Area= 300cm2

A Bale

400 N

Small piston A =50 cm2

R

(i)    Calculate the force F exerted by small piston on the liquid due to the lever. 												(4mk)

(ii)   Find the weight of the Bale supported by the large piston.  		 (4mk)

(iii) 	State two properties of the liquid used as the brake fluid		  (2mk)

- Figure shows a hydraulic press system using a lever of negligible mass on the side of a small piston pivoted at point P. A force of 100N is applied at R.

100 cm

50 cm

Liquid

Area = 180cm2

2

A Bale

100 N

50 cm2

Fixed

P

R

(i)    Calculate the force F exerted by small piston on the liquid.(2 mk)

(ii)   Find the weight of the Bale supported by the large piston.(2 mk)

- The figure shows a device for closing a steam outlet. The area of the piston is 4.0 x 10-4m2 and the pressure of the steam in the boiler is 2.0 x 105Nm-2. Determine the weight W that will just hold the bar in the horizontal position shown.

15cm

Pivot

45cm

Steam

W

- A see – saw of length 5m is pivoted at the centre. A student of mass 50kg sits at one end and is balanced by another student of mass ‘m’ sitting at a distance of 1m from the other end. Calculate the value of ‘m’

- A mass of 100g is placed on a 20cm mark and a mass of 50g on a 40cm mark of a uniform metre rule which is balanced at its centre. Where should a further 100g mass be placed to balance the arrangement?

- A meter rule of negligible weight is balanced by masses of 24N and 16N suspended from its ends. Find the position of the pivot.

- A half meter rule is balanced at the 15cm mark when a load of 1.2N is hung at the zero mark. Calculate the weight of the half meter rule.	(3mk)

- Two men P and Q carried a uniform ladder 3.6 m long weighing 1200N. P held the ladder from one end while Q supported the ladder at a point 0.4m from the other end. Calculate the load supported by each man.

- A 20m uniform plank AB of mass 20Kg is put on a wedge such that it does not balance horizontally. Three pupils of mass 50Kg, 35Kg and 30Kg sit on the plank at a distance 3m, 7m and 18.5m respectively from A. How far must the wedge be placed from A for the arrangement to balance horizontally?

(3 mk)

- A uniform half metre rule of mass 37.5g is freely pivoted at 10cm mark. At what mark should a body of mass 75g be suspended for the system to balance

- A uniform metre rule is balanced at the 20cm mark when a load of 1.2N is hung at the zero mark. Calculate the weight and mass of the metre rule.											(3mk)

- A uniform metre rule pivoted at its 15cm mark is balanced by a 200g mass suspended at the 5cm mark. Determine the weight of the metre rule.

- A mass of 100g is placed on a 20cm mark and a mass of 50g on a 40cm mark of a uniform metre rule which is balanced at its centre. Where should a further 100g mass be placed to balance the arrangement?	(3mk)

- Two men P and Q carried a uniform ladder 3.6 m long weighing 1200N. P held the ladder from one end while Q supported the ladder at a point 0.4m from the other end.

(i) 	Sketch a diagram showing the forces acting on the ladder.

(ii) 	Calculate the load supported by each man.

- A uniform half- metre rod is balanced by a weight of 38N at one end. If the pivot is placed 10cm from the same end, calculate the weight of the rod.

- With the metre rule remaining on the knife-edge at the 30 cm mark, a mass of 125g is suspended from the 70 cm mark. The mass of 500g is moved until the rule is balanced. Determine the new position of the 500g mass

SCHEEM

Explain why it is difficult to steer a bicycle by gripping the centre of the handlebars.

The turning effect (moments) will be smaller ; due to  a shorter perpendicular distance  of the line of action. Of force applied from the pivot;

The figure below shows a uniform metre rule of weight 1.0N suspended from spring balances. A load is attached to the extreme right hand end C. The spring balance attached to the extreme left hand end of the rule (A) reads 0.25N. The spring balance attached at B a distance X from the right hand end reads 1.25N.

0.25N

1.25N

A

B

L

C

X

(i)	Calculate the weight of load L				(2mks)

Sum of downward forces = sum of upward forces

1.0   +   L   = 0.25   +  1.25  ;

L   = 0.25  + 1.25 – 1.0

L   = 0.5 N ;

(ii)	Determine the value of distance X by taking moments about A    (3Mks)

Taking moments about A

Sum of clockwise moments = sum of anti- clockwise moments

clockwise moments  = ( 0.5   x  1 + 1 x 0.5  ) ;

= 1 Nm

Anticlockwise moments = ( 1 – x ) 1.25 ;

= ( 1.25  - 1.25 x )  Nm

1 = 1.25  - 1.25 x  ;

x =       0.25

1.25

x = 0.2m  ;