- State Hooke’s law		(1mk)

- Define the term elastic limit, as used in stretching of materials	(1mk)

- State the SI units of elastic constant of a spring                            (1mk)

- State the features that govern the strength of a spiral spring of a given material.	        										 (2mks)

- State two factors on which the extension of a wire depends on assuming it obeys Hooke’s Law.						(2mks)

- Apart from the diameter and length, name another factor that determine the spring constant of a spiral spring.							(1mk)

- A heavy load is suspended on a wire.  Give any one factor that will determine extension in the wire. 							(1mk)

- Distinguish between ductile end and brittle material

- It is easier to bend an iron rod than a glass rod of the same dimensions at room temperature. Give a reason for this

- A spiral spring stretches by 0.6 cm when a mass of 300g is suspended on it. What is the spring constant?

- An unloaded spring has a length of 15cm and when under a load of 24N it has a length of 12cm.What will be the load on the spring when length is 10cm?

- An object of weight 20N attached at the end of a spring causes an extension of 0.5 cm on the spring.

- Determine the spring constant.				(2mk)

- Determine the weight of an object that would cause an extension of 0.86 cm on the same spring.					(1mk)

- A copper wire is 2m long. A force of 4N suspended on the wire while other end is fixed increases its length to 2.001m. What force would make the length of the wire 2.032m?  							(3mks)

- The pointer of an unloaded spring reads 32 cm. when a mass of 120g is applied to the spring, the pointer reads 38cm. a pan in which a mass of 210g is placed is now hang from the spring and the pointer reads 48cm. determine the mass of the pan.					(4mks)

- Two identical helical springs are connected in series. When a 50g mass is hang at the end of the springs, it produces an extension of 2.5 cm. Determine the extension produced by the same mass when the springs arc connected in parallel.								(3mks)

- Two springs  of negligible weights and spring constants  50N/m and 75N/m respectively are connected in series and suspended from a fixed point. Determine the total extension when a mass of 7.5kg is hung from the lower end.	 (3mk)

- Two springs of negligible weight and of spring constants 100Nm-1respectively are connected end to end and suspended from a fixed point. Determine;

- The total extension when a mass of 7.5 kg is hung from the lower end.  (2mk)

(ii)	The elastic constant of the combined of springs.		(2mk)

- A spring extends by 6cm when supporting a mass of 0.06kg on earth. When the spring is used to support the same mass on the moon, it extends by 1 cm. determine the moons gravitational strength. (take the gravitational field strength on earth as 10N/kg) 						(3mk)

SPRINGS

- Fig below shows a stretched spring held in position by two identical spring balances P and Q.

P

Q

10N

What will be the reading on spring balance Q? 					(1mk)

- The figure below shows a spring balance. Its spring constant is 125Nm-1. The scale spreads over a distance of 20cm.

20cm

Scale

Determine the maximum weight that can be measured using this spring.   3mk

- The identical springs of spring constant 3N/cm are used to support a load of 30N as shown.

30N

Determine the extension on each spring    		       (3mks)

- The spiral springs shows in the figure below are identical. Each spring has a spring Constant K=100N/m Determine the total extension caused by the 150N weight.

150N

(3mk)

- Figure shows a mass of 12g suspended on a set of 6 identical springs. When the mass was hanged on spring A, it extended by 5cm.

12g

D

E

F

B

C

A

Determine the extension of the combination shown if each spring and rod has negligible weight.									 (3mks)

- The diagram below shows a force of 480N hanged on a set of 6 identical springs. Each of the spring has elastic constant 100N/m.

480N

C

B

A

D

E

F

480N

C

B

A

D

E

F

Determine

(i)  The extension of springs A, B, C						(2mk)

(ii)  The extension of springs D, E							(2mk

(iii)  The extension of spring F. 							(2mk

(iv)  The total extension of the system						(2mk)

- Three identical springs each of spring constant 10N/M and weight 0.5N are used to support a load as shown. Determine the total extension of the system

C

A

B

6N

(2mk

- (i)     A light spring fixed at one end extends by 2.0cm when a weight of 120g is suspended from the free end .Use this information to answer question 7 and 8.

(ii)     Find its spring constant.						(1mk)

(iii)     Three such springs are arranged in a manner shown in figure 5 below and used to support  a load If each extends by 2.0cm, what is the weight of the load?										(2mks)

X

- Two springs of negligible weight and elastic constants of 800N/m and 400N/m are arranged as shown below with a load of 160N supported at the bottom. Calculate the total extension of the springs.

160N

- Three identical springs A, B and C are used to support a 15.5N weight as shown in the figure below.

A

C

B

15.5N

If the weight of the horizontal beam is 0.5N, determine the extension of each spring given that 4N causes an extension of 1cm.	    				(3mks

- Three identical springs, each of mass 20g, are joined together as shown below and support a mass of 100g.

100g

If the spring constant of each spring is 500N/m, calculate the combined spring constant of the set-up. 								(3mk)

- Study the figure below and answer the question that follows.

A

C

B

50N

D

The springs A, B, C and D are identical and each extends by 2cm. When a force of 50N is suspended from it. Determine the extension of the system.(3 mk)

- The diagram below shows three identical springs which obey Hooke’s law.

Determine the length X.						 (3 mk)

- Figure shows a spiral spring fixed on a bench vertically. A mass of 0.5kg is placed on top as shown in the figure below.

6cm

4cm

0.5kg

(a)

(b)

In (a) the height of the spring is 6cm while in (b) the height is 4cm. find the energy stored in the spring in figure (b)					(2mk

- The diagram below shows the same spring in three different situations; when unloaded, when supporting a load of 400g and when supporting a wooden block. Find the mass of the wooden block. 			(3mk)

5cm

Block

400g

6.8cm

8cm

Unloaded spring

3.5cm

Block

150g

4.8cm

6cm

Unloaded spring

The diagram in figure below shows unloaded spring, when supporting a load of 150g and a wooden block.

Find the mass of the wooden block.						(3mks)

- Figure shows two identical springs arranged in parallel supporting  a force F. The spring constants for the parallel arrangement is Kp while K1 for each spring Show that Kp= 2K1                                                                   3mk

K1

K2

F

GRAPH SKETCHES

- Figure below shows the variation of force with extension for a certain spring.

Force (N)

Extension (m)

On the same axis, sketch the variation of force with extension for another similar spring whose length is double the first spring	      (1mk)

- Figure shows the variation force against extension for a spring obeying Hooke’s law

Force (N)

Extension (m)

Sketch the same axes the variation of force against extension when two such springs are connected end to end.							(1mk)

- The figure below shows two springs A and B made of same material, same diameter and same length but with different number of turns.

A

B

Load

Load

On  the axis below sketch a graph of load against extension for each. (Hooke’s law is obeyed)									(1mk)

Force (N)

Extension (m)

- The graph in the figure below was obtained when suitable weights were suspended from a spiral spring and extensions measured

  - Explain the shape of the graph between

- AB

- CD

  - Two identical spiral springs are then arranged in parallel and the weights suspended. on the same axes above, sketch the graph that would now be obtained 	       							(2mks)

- The graph in the figure below was obtained when suitable weights were suspended from a spiral spring and extensions measured	.

8

2

4

6

10

0

80

160

240

1200

40

200

Force (N)

Extension (cm)

B

A

C

- Explain the shape of the graph between

- AB										(1mk)

- BC											(1mk)

From the graph:

- Determine the elastic constant of the spring used.			(3mk)

- What is the elastic limit of the material?					(1mk

- Sketch on the same axis the graph of force against extension when two such springs are connected in parallel.					(1mk)

- The graph below was obtained in an experiment to investigate the stretching of materials.

8

0

2

4

12

6

10

0

80

160

240

1200

40

200

Force (N)

Extension (m)

(i) Determine the constant of the spring used.			(2mk)

(ii) Determine the elastic limit of the material.			(1mk)

- Use the graph below showing the variation of the length of a spring and the compressional force applied on it to answer questions 3 and 4.

Compression Force (N)

Length of spring (m)

A

O

B

(i)	State the significance of the point labelled A.                        (1mk)

(ii)	Explain the nature of the graph between A and B.                (1mk)

- An experiment was performed to find out how the length L of a spiral spring varies with the compression force, F The figure shows the variation.

20

Force (N)

L (cm)

A

B

40

60

80

8

4

2

6

- Draw a diagram of a possible set up of the apparatus

- Over which range of the force does the spring obey Hooke’s law?

- Suggest a reason for the shape of the graph between 20N and 80N.

- The graph (curve) below show the variation of force against extension (cm) of two spiral springs of same material , same wire thickness, length but of different diameters (one large and the other small).Identify which graph (A or B) represents which spring.				(2mks)

Force (N)

Extension (m)

0

6

12

2

8

4

10

20

40

60

B

A

TABLES

- The table below shows the value of extensions of a spiral spring when various forces are applied.

| Force F(N) | 0 | 1.0 | 2.0 | 3.0 | 4.0 | 5.0 | 6.0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Extension e(cm) | 0 | 0.8 | 1.6 | 2.4 | 3.2 | 4.0 | 4.8 |

- Plot a graph of force ( y – axis )  against the extension.	(5mk)

- Determine the work done in stretching the spring by 2.7cm; using the graph.									(2mk)

- Determine the spring constant of the spring. Give your answer in SI units.												(2mk)

- The table below shows the values of extensions of a spiral spring when various forces were applied on it.

| Force, F (N) | 0.0 | 1.2 | 2.0 | 3.6 | 4.0 | 4.8 | 6.0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Extension, e (cm) | 0.0 | 0.8 | 1.3 | 2.5 | 2.7 | 3.2 | 4.0 |
| Extension (e) (m) |  |  |  |  |  |  |  |

i)	Plot a graph of extension in metres (y-axis) against force (x-axis).	 (5mks)

Use the graph to determine the spring constant of the spring.	         (2mks

(ii) Determine the work done in stretching the spring by 4.5cm.	          (2mks)

- The following results were obtained in an experiment to verify Hooke’s law when a spring was extended by hanging various loads on it.

| Load, L(N) | 0.00 | 1.00 | 2.00 | 3.00 | 4.00 | 5.00 | 6.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Length of spring (cm) | 10.00 | 11.50 | 13.00 | 14.50 | 16.00 | 18.50 | 24.00 |
| Extension, e (cm) | 0.00 |  |  |  |  |  |  |

i)	Complete the table for extension, e, above		(1mk)

ii)	Plot a graph of Load (y-axis) against extension.		(5mk)

ii)	From the graph, determine the spring constant, k.	(3 mk)

- A student carried out an experiment to investigate the relationship between the force and extension produced on a spiral spring. He tabulated his results as shown below.

| Force (N) | 0 | 0.8 | 1.5 | 3.0 | 4.5 | 6.0 | 7.5 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Extension (cm) | 0 | 0.50 | 1.0 | 2.0 | 3.0 | 4.0 | 5.0 |

(i) Plot a graph of extension in cm in the y-axis against force in N	(5 mk)

(ii) Determine the spring constant.				 (3 mks)

(iii) What force would be required to produce an extension of 2.5cm	(1 mk)

(iv) What extension is produced by:

(a) A force of 5.5N							(1 mk)

(b) A mass of 700g							(1 mk)

- The table below has a data recorded for a compressional force on a spring balance.

| Force F (N) | 0 | 5.0 | 10 | 15 | 17.5 | 22.5 | 35.0 | 40 | 4 | 50 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of spring L (cm) | 16.5 | 15.0 | 13.5 | 12 | 11.6 | 9.75 | 8.25 | 8 | 8 | 8 |
| Compression of spring e (cm) |  |  |  |  |  |  |  |  |  |  |

i)    Complete the table for the values of compression (e)		(2mk)

ii)    Plot a graph of extension (vertical axis) versus force applied on the spring

(horizontal axis). 									(5mk)

From your graph determine.

iii)   The spring constant K.						(2mk)

iv)   Work done in compressing the spring until the coils just come into contact.									(2mks)

- A spring with its upper end fixed, hang vertically and several masses are suspended from its lower end one at a time.  The readings were recorded as shown.

| Mass in kg | 0 | 0.02 | 0.04 | 0.06 | 0.08 | 0.10 |
| --- | --- | --- | --- | --- | --- | --- |
| Extension (mm) | 110 | 121 | 129 | 139 | 151 | 161 |
| Force N |  |  |  |  |  |  |
| Extension in m |  |  |  |  |  |  |

i)  	Fill in the table           							 (1mk

ii)  	Plot a graph of extension in (m) against force in N.  		(5mk)

- (i)  From the graph determine the extension of a mass 0.045kg. Give your

answer in mm  									  (2mk)

(ii) Determine the spring constant of the spring 		(2 mk)

d)  	If two such springs were connected in series what extension would they show when a mass of 1.5kg hangs from one end? 		         (3mks)