- What is meant by;

(i) Streamline flow								(1 mk)

(ii) Turbulent flow								(1 mk)

- State Bernoulli’s principle.							(1 mk)

- State one assumption made in Bernoulli’s fluid flow.

- Give three examples of Bernoulli’s effect in air.		(3 mk)

- State any three properties of an ideal fluid that obeys Bernoulli’s principle											(3mk)

- State the equation of continuity. Define any symbols used.	(1 mk)

- In deriving the equation of continuity, what three assumptions are made?

- An oil drop of volume V m3 introduced on the surface of water spreads to forma patch whose area is a m2.  Derive an expression for obtaining the diameter, d of a molecule of oil.					(2mk)

- The figure below shows an incompressible fluid flowing through a pipe, A1 and A2 are the cross-sectional areas of the pipes in the larger section and smaller section of the pipe respectively, while V1 and V2 are speeds of the fluid at the two sections of the pipe.

A1

A2

V2

V1

Derive an expression for the ratio of the speeds   V2 V1  in terms of A1 and A2

- State how the pressure in a moving fluid varies with speed of the fluid.

(2mk)

- Explain one danger of Bernoulli’s effect 		             (2mks)

- Explain why a fast moving car appear lighter		     (1mk)

- A horse rider bends forward when the horse is on speed.  Explain.	(2mk)

- Give two examples of devices in which Bernoulli’s effect is applied.   (2mk)

- It is observed that a vehicle which has a door at the rear side gathers more dust inside when moving at high speed on a dusty road with the door open than the one which has an open door on the side. Explain this observation												(2mk)

- Water flows in a horizontal smooth pipe. State the changes that would be observed in the nature of the flow if the speed of the water is steadily increased from low to high value?

- Two table Tennis balls are in the same level while suspended from   threads a short distance apart. A stream of air is blown between the balls in a horizontal direction. Explain what happens to the balls

- When spraying a field of water using a hose pipe, it is common to reduce the pipes opening in order to spray water furthest. Other than pressure, what other quality is varied in the process?

- State how the pressure in a moving fluid varies with the speed of the fluid.

- It is dangerous to stand close to a road when a fast moving lorry passes. Comment on this statement							 (1mk)

DIAGS

- The passengers in a bus would complain about smoke if the driver smokes when in the driver’s seat and the bus is stationary but not when the bus is moving. Explain.			(2mk

- The diagram shows a cross-section of an aeroplane wing when the aero-plane is moving at constant height  and constant speed, an upward force equal and opposite to its weight is exerted on its wing.

Direction of

force

Upward force

Wing

(a)	What is the cause of the upward force?			           (2mks)

(b)	Why is the shape of the wing crucial in producing this upward force?

- The figure below shows an aerofoil.

X

Aerofoil

X

Explain why it lifts in the direction X indicated.   	(2mks)

- “Air flow over the wings of an aircraft causes a lift.  Explain this statement with an aid of a well labelled diagram.					(2 marks)

- Fig below shows streamline of a fluid flowing from left to right of the object.  Point Y, is on the lower side of the object.

Y

OBJECT

State with reason, the side of the object on which the velocity of flow of the liquid is higher.

- The figure below shows two balls suspended from threads a short distance apart.

Thread

Ball

A stream of air is blown between the balls in a horizontal direction.

  - State the observation                                                       (1mark)

  - Explain the observation                                                      (2marks)

Air blown

Styrofoam

A piece of Styrofoam rests at the bottom of a test tube as in figure 2 below. Explain what is observed when air is blown across the mouth of the test tube.											(2 mks)

- The figure below shows a light body floating in a container

Air

a) 	State and explain the observation when a stream of air is blown over the mouth of the container as shown						 (2mks)

b) 	Explain why the base of the cylinder is wider 				(1mk)

- Figure below is a manometer containing water. Air is blown across the mouth of one tube and the levels of the water changes as shown.

Blow air

Explain why the level of water in the left limp of manometer is higher.(2 mk)

- A pupil blows a current of air over the surface of a sheet of paper held close to its mouth. State and explain what happens to the paper.

- A stream of air is passed over a piece of paper as shown below.

Blow Air

Paper

State and explain the observations made.				(2mks)

- The figure 4 below shows two light sheets of paper arranged as shown.

Q

R

State what is observed if strong air is blown along the lines shown at the same time behind paper Q and in front of paper R as shown. 		(1mk)

- Figure shows a whistle.

Observer

Air

Pith ball

When air is blown the sound produced varies. Explain this observation using Bernoullis Principle									(2mks)

- Water is made to flow at a high speed in a uniform horizontal tube fitted with vertical tubes X, Y and Z of same cross -sectional area

Water flow

X

Y

Z

(i) Indicate the possible water levels in the vertical tubes                (1mk)

(ii) Give a reason for your answer in part (i) above                   (1mk)

- The diagram below shows water in a long pipe with parts A and B as shown

A

B

(i)  	Show on the diagram, the likely levels of water in section A and B  												(1mk)

(ii)  Explain your answer  						 (1 mk)

- Figure shows air flowing through a pipe of non-uniform cross-sectional area. Two pipes A and B are dipped into liquids as shown.

Air flow

Liquid

Liquid

A

B

a)	Indicate the levels of the liquids in Pipe A and pipe B.			(1mk)

b)	Explain your answer in above.							(1mk)

- The figure below shows air flowing through a pipe of different cross-section areas. Two pipes A and B are dipped into water.

Air flow

Water

A

B

Explain the cause of the difference in the levels of water in the pipes A and B. (2mk)

- In the figure, below, air is blown into the tube. Indicate the level of water in the beaker X and Y								(1mk)

Air flow

Liquid

Y

X

- Water flows in a pipe of varying diameter as shows in figure 9 below.

Hole A

Hole B

Flow of water

- Two perforations developed with time at points A and B as shown. If the holes are of same dimensions, suggest with a reason which hole leaks more.(2mk)

- If the diameters of the larger end of the pipe in figure above is twice that of its smaller end, find the percentage decrease in speed as the water flows from narrow to wider end. 						(3mk)

- Figure below represents a tube through which liquid is flowing in the direction shown by the arrow.

X

Y

Z

On the diagram show the relative position of the level of the liquid in sections marked x, y and z.

- Figure  shows a tube of varying cross sectional area. V1V2V3 and V4 represents the velocities of water as it flows steadily though the sections of the tube.

V4

V2

V3

V1

Arrange the velocities V1, V2,V3 and V4 in descending order.

(b) On the same diagram show the path followed by each body                       (2mks)

- The figure below shows a pith ball being lifted in to a funnel end of a blower.

Explain this observation 						     (2 mk)

- Figure shows a ball bearing resting at the top of a barrel of a bic pen.

Air

Barrel

Ball bearing

State and explain the observation made when air is blown into the barrel.

(2mks

- The figure below sows a Bunsen burner.

Gas

Gas tap

Air

Nozzle

Explain how air is drawn into the burner when the gas tap is open. (2mk)

- The diagram below shows a bursen burner

Gas in

Barrel

Hole

Nozzle

Explain how air is drawn into the barrel 				( 2mk)

- Figure below shows a modern latrine.

Exhaust pipe

Pit

Explain how the exhaust pipe minimizes foul smell especially on a windy day.  (2mks)

CALCULATIONS

- The diagram below shows water flowing steadily in a tube of varying cross-sectional area.  Study it and answer the question that follows:

120cm2

40cm2

If the velocity of the water in the wider section is 0.4ms-1 what is the velocity in the narrower section?								(2mks)

- Water flows through a horizontal pipe of varying cross-section area as shown

v = 2m/s

Pipe

100cm2

B

60cm2

A

in figure below.

The velocity of water in pipe A is 2m/s

(i)  Determine the velocity of water in pipe B.				           3mks

(ii)  State two assumptions taken in Bernoulli’s  fluid flow.			  2mks

- A pipe of radius 3mm is connected to another pipe of radius 9mm. If water flows in the wider pipe at a speed of 2ms-1, what is the speed in the narrower pipe				(3mks)

- Water flows steadily through a pipe of radius 25 mm at 20 ms-1. The pipes narrows at some point and its radius reduces to 10mm. what is the speed of water at this point?

- Water flows through a pipe of varying cross-sectional area, one end of the pipe is 5cm2 and water flows at that point at a velocity of 0.3m/s. calculate the cross-sectional area of the other end if water flows at that end at 1ms-1. (3mk)

- Water flows along a horizontal pipe of cross sectional area 48cm2 which has a constriction of  cross sectional area 12cm2 at one place. If the speed of the water at the constriction is 36m/s, calculate the speed in the wider section.											(3mk)

- Water flows steadily through a pipe of diameter 30 mm at 20 ms-1. The pipes narrows at some point and its diameter reduces to 20 mm. what is the speed of water at this point?  (3mk)

- Water flows  through  the horizontal  pipe  at the rate  of 400 cm3/s . what is the speed  of the water  if the diameter  of the pipe  is 1.4 cm? ( 2 marks)

- Water flows along a horizontal pipe of cross – sectional area 30cm2 at a speed of 40ms-1. It reaches a section where the speed is 7.5ms-1. Find the area of this section of the pipe. (2mk)

- A liquid flows along a horizontal pipe of cross-sectional area 20cm2 with a speed of 5m/s. The speed increases to 8m/s where there is a constriction. Calculate the cross- sectional area of the constriction. 	(2mk)

- Water flows through a pipe with varying cross sectional area. The narrower section has an area of 5cm2 while the wider section has an area of 15cm3. If the velocity of the water trough the narrow section is 20m/s, determine the velocity in the wider section.					(3mk)

- A heart pumps blood at a rate of 1.8 x 103 cm3/ min though its aorta is of cross sectional area 0.6 cm3. The blood spreads into a capillary network that is equivalent to about 4 x 106 fine tubes each of diameter 6 x 10-3cm.

Calculate:

  - The average velocity of blood in the  aorta

  - The average blood velocity in the capillary tubes.

- Water flows along a horizontal pipe of cross sectional area 30cm2. The speed of the water is 4m/s but it reaches 7.5m/s in a constriction in the pipe. Calculate the area of the constriction.

- Water flows in through a horizontal pipe of cross – sectional area 100cm2. At the outlet section the cross- sectional area is 5 cm2 . If the velocity of water at the larger cross- section is 1.25m/s

(i) Find the volume of water leaving the pipe in the second	( 2mk )

(ii) Calculate the velocity of water in the smaller pipe	2mk )

- A horse pipe of internal diameter 4cm is connected to a sprinkler with 25 holes each of diameter 0.004 cm, the water in the pipe flows at a speed of 5 cm/s. Determine the velocity with which the water leaves the sprinkler. 				 							(3mk)

- A lawn sprinkler has 20 holes each of cross-sectional area  is connected to a hose pipe of cross-sectional area 2.4cm2. if the speed of water in the hose pipe is 1.5 m/s.

- Calculate the flow rate in the hose pipe.	(3mk)

- The speed of water as it emerges from the holes. (3mks)

- The mass flux in the hose pipe. Take density of water 1000kg/m3.

(3mk)

- A lawn sprinkler has 20 holes each of cross-sectional area 3 x 10-2 cm2 is connected to a hose pipe of cross-sectional area 4cm2. if the speed of water in the hose pipe is 3.6 m/s.

    - Calculate the flow rate in the hose pipe. 			(3mk)

    - The speed of water as it emerges from the holes. 		(3mk

    - The mass flux in the hose pipe. Take density of water 1000kg/m3.

(2mk

(d)	A lawn sprinkler has 20 holes each of cross-sectional area 9 x 10-2 cm2 is connected to a hose pipe of cross-sectional area 6cm2. if the speed of water in the hose pipe is 48 m/s.

    - Calculate the flow rate in the hose pipe.			(3mk)

    - The speed of water as it emerges from the holes. 		(3mk

    - The mass flux in the hose pipe. Take density of water 1000kg/m3.

(2mk

- A pump draws water from a tank and issues it from the end of a hosepipe which is 2.5 vertically above the level from which the water is drawn. The cross –sectional area of the hosepipe is1.0 x 10-3m2 and the water leaves the end of the hosepipe at a speed of 5m/s. Calculate the power of the pump. (density of water = 1000Kg)

SCHEEM

- A horse rider bends forward when the horse is on speed.  Explain.	(2mk)

To attain streamline position and reduce air resistance.

- It is observed that a vehicle which has a door at the rear side gathers more dust inside when moving at high speed on a dusty road with the door open than the one which has an open door on the side. Explain this observation												(2mk)

When stationary the smoke moves around indiscremetely and the passengers get irritated but 	when the bus is moving there exists a pressure difference due to the outside air moving fast so the 	smoke moves out of the bus.  1

- “Air flow over the wings of an aircraft causes a lift.  Explain this statement with an aid of a well labelled diagram.					(2 marks)

NB:	Lines on the upper side must be close.

Lines on the lower side must be wider.